|  | AIC Inc. No. 152, Sec.4, Linghang N. Rd, Dayuan District, Taoyuan City Taiwan 337 T: +886-3-433-9188 F: +886-3-287-1818 www.aicipc.com |
| --- | --- |

Design Validation Test Report

Reliability Test

SB202-SU / XP1-S202SUXX

| Date |  |  | 2025 / 10 / 27 |  |  |
| --- | --- | --- | --- | --- | --- |
| Doc. Number |  |  | RD251027A29 |  |  |
| Doc. Version |  |  | A |  |  |
| Number of Pages |  |  | 15 |  |  |
| Test Report Perform By |  |  | DQA Team |  |  |
| Approved By |  |  | Reviewed By | Tested By |  |

© 1996-2025 AIC Inc. All rights reserved.
All other trademarks are the property of their respective companies.
CONFIDENTIALITY NOTICE: The information contained in this document is confidential, and may not be used, published or redistributed without the prior written consent of AIC Inc.

Table of Contents

1.	Revision History	2

2.	Introduction	2

2.1.	Purpose	2

2.2.	Scope	2

2.3.	UUT Configuration	2

2.4.	Test Equipment and Software	3

3.	Summary	4

3.1.	Test Item Result	4

3.2.	Test Conclusion	4

3.3.	Bug List	4

3.4.	UUT Photo	5

4.	Reliability Test Result	6

4.1.	Environmental Qualification Test	6

4.1.1.	Non-Operational Storage Test	6

4.1.2.	Operational Temperature Cycle Test	7

4.1.3.	Cold / Hot Start Test	8

4.2.	Mechanical Structure Test	9

4.2.1.	Operating Vibration Test	9

4.2.2.	Non-operating Vibration Test	10

4.2.3.	Operating Shock Test	11

4.2.4.	Non-Operation Square WaveShock Test Report	12

4.3.	Packaged Test	13

4.3.1.	Package Test	13

4.4.	Other Test	15

4.4.1.	Rotational Vibration Index Test	15

- Revision History

| Revision | Description | Date | By |
| --- | --- | --- | --- |
| A | Initial version | 2025.10.27 | King.Huang |

- Introduction

  - Purpose

This document is the DVT Test Report for the SB202-SU (XP1-S202SU02E01), the system’s configuration includes Sirius Main Board, GENUINE INTEL(R) XEON(R) Q3J1 1.2Ghz 350W *2pcs, Micron 16GB DDR5 RDIMM MTC10F1084S1RC56BD1*32 pcs, 3.5” Seagate 4TB SATA HDD ST4000NM024B*8pcs, Seagate 22TB SATA HDD ST22000NM0001E*8pcs(only for RVI Test),

2.5” WD SN650 Nvme SSD 7.68TB WUS5EA176ESP5E1*4pcs, 2.5” Seagate 2.5” 1TB SATA HDD ST1000NX0313*2pcs, M.2 (OS) Crucial P3 Plus 500GB SSD CT500P3PSSD8 *1pcs, SUNON Fan Module *4pcs and AcBel Power Module*2pcs.

  - Scope

To ensure system level products are capable of withstanding the varied environmental condition that may occur during operation/non-operation, and identify the reliability has been designed-in the product. The test specification and configuration are approved by PJM and customer to take precedence for test execution.

  - UUT Configuration

The table summarizes the Key part of UUT’s configuration for this test report as below.

Table 1: List of Test UUT’s configuration

| No | Key Part | Manufacturer | PN/Revision. | S/N |
| --- | --- | --- | --- | --- |
| 1 | Mother Board | AIC | BMB-DPG0001AC01 | 422-25062800100012 |
| 2 | Backplane Board | AIC | BBP2FB20001AC01 | 422-24071701100018 |
|  |  |  | BBP2HD10001AB02 | 422-24081400300055 |
| 3 | RISER CARD | AIC | BRC2PE20052AC01 | 422-24111100600006 |
|  |  |  | BRC2PE20053AC01 | 422-24111100200113 |
| 4 | Fan Module | SUNON | VG80561BX-Q140-S9H | NA |
| 5 | Power Module | AcBel | R1CA2202E | A00245200162000AGBFSM005 |
|  |  |  |  | A00245200162100AGBFSM005 |

  - Test Equipment and Software

The following tables summarize all test equipment and software required for this test report. This information is also provided in the test condition table of each test case.

Table 2: List of Test Equipment

| No | Test Equipment | Manufacturer | Model | Calibration Date | Due Date |
| --- | --- | --- | --- | --- | --- |
| 1 | Vibration Tester | KING DESIGN | KD-9363-EM-1000- F2K-50N250 | 2025/03 | 2026/03 |
| 2 | Drop Tester | KING DESIGN | KD-128A | N/A | N/A |
| 3 | KSON Chamber | KSON | THS-D4T-100 | 2025/01 | 2026/01 |
| 4 | Walk-in Chamber | KSON | ATH-Z600 | 2025/08 | 2026/08 |

Table 3: List of Test Item with Software

| No | Test Item | Test Software | Vender | Version | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | Non-Operational Storage Test | Iometer | Intel | 1.1.0 |  |
| 2 | Operational Temperature Cycle Test | PTU | Intel | 4.7.0 |  |
|  |  | Iometer | Intel | 1.1.0 |  |
|  |  | Memory Test | N/A | N/A |  |
| 3 | Cold / Hot Start Test | QT Cycling Multi | AIC | N/A |  |
| 4 | Operating Vibration Test | Iometer | Intel | 1.1.0 |  |
| 5 | Non-Operating Vibration Test | Iometer | Intel | 1.1.0 |  |
| 6 | Operating Shock Test | Iometer | Intel | 1.1.0 |  |
| 7 | Non-Operational Square Wave Shock Test | Iometer | Intel | 1.1.0 |  |
| 8 | Packaged Test | Iometer | Intel | 1.1.0 |  |
| 9 | Rotational Vibration Index Test | Iometer | Intel | 1.1.0 |  |

- Summary

  - Test Item Result

| No | Test Item | Date | Results | Remark |
| --- | --- | --- | --- | --- |
| 1 | Non-Operational Storage Test | 2025/08/30~2025/09/11 | Pass |  |
| 2 | Operational Temperature Cycle Test | 2025/10/17~2025/10/20 | Pass |  |
| 3 | Cold / Hot Start Test | 2025/10/21~2025/10/27 | Pass | Use RD solution can pass it. |
| 4 | Operating Vibration Test | 2025/10/15~2025/10/16 | Pass |  |
| 5 | Non-Operating Vibration Test | 2025/10/15~2025/10/16 | Pass |  |
| 6 | Operating Shock Test | 2025/10/15~2025/10/16 | Pass |  |
| 7 | Non-Operational Square Wave Shock Test | 2025/08/12~2025/08/12 | Pass |  |
| 8 | Packaged Test | 2025/08/21~2025/08/22 | Pass |  |
| 9 | Rotational Vibration Index Test | 2025/07/22~2025/10/14 | Pass | Use RD solution can pass it. |

  - Test Conclusion

There is no abnormal in system function and physical damage during the test. The UUT can pass

these tests.

  - Bug List

| Bug No. | Severity | Description/Execution Action | Status | Issue Owner | Remark |
| --- | --- | --- | --- | --- | --- |
| #10083 | High | After the Cold / Hot Start Test, there P0, P1 VR Inlet Over temperature issue, RD solution is update BMC Version to 4A00SIR011202 for verifies, this test result is pass. | Close | BMC/Jethro Yeh |  |
| #10082 | High | After the RVI test, the 3.5” HDD performance has drop over 15%, RD solution are add sound-absorbing cotton at the fan inlet, use these solution can pass it. | Close | ME/Jack Weng |  |

  - UUT Photo

SB202-SU (XP1-S202SU02E01)

Top View

- Reliability Test Result

  - Environmental Qualification Test

    - Non-Operational Storage Test

      - Description

To ensure that there are no adverse effects to electrical function, mechanical structure or appearance by sustained storage and shipping temperature of extremes.

      - Test Procedure

- Pre-testing of the UUT shall test for electrical / mechanical functionality and appearance.

- Run diagnostics for a period of 2 hours on the systems before test.

- Record all test results and photographs for anomalies.

      - Pass/Fail Criteria

- Visual inspection was passed without any abnormal or above failure mechanisms:

- HDD/SSD tray deformation check

- Chassis deformation check

- Functional was passed without any failure or abnormal symptom:

- BMC SDR /SEL have no any abnormal record or status detect failure.

- Diagnostics (Iometer) have no any failure (error) occur

- All install device can be detected before and after test (device check by OS)

- No any subassembly noise.

- Without corrode

- The R.T.C specification is +/- 2 second per day. (Maximum allowable +/- 10 second per day).

      - Result

| Test Result | Before Test | After Test |
| --- | --- | --- |
| Physical Damage | Pass | Pass |
| Functional Check | Pass | Pass |

| Test Result | GOV.TIME | SYSTEM.TIME | TIME DIFFERENCE |
| --- | --- | --- | --- |
| RTC Check | 2025/09/11 PM:16:22:36 | 2025/09/11 PM:16:22:30 | Delay 6s |

  - Mechanical Structure Test

  - Packaged Test

  - Other Test